Wireshark DNS & HTTP Network Traffic Analysis

Cybersecurity Portfolio Project

OLAMIDE |  SOC Level 1 Candidate  |  Capture Date: 30 March 2026

# Project Overview

This project involves capturing and analysing real network traffic using Wireshark on a Windows machine. The focus is on DNS (Domain Name System) and HTTP traffic — understanding what devices are communicating, what services are being accessed, and what this means from a security analyst's perspective.

# Tools Used

- Wireshark (Windows)

- Capture filters: dns, http, tcp.stream eq 17

- Capture file: wireshark_WiFiTHI3M3.pcapng

# Environment

| Detail | Value |
| --- | --- |
| Network Type | Home WiFi |
| Capture Interface | WiFi (NPF) |
| Client Device | Windows Laptop — 172.20.10.5 |
| Gateway/Router | 172.20.10.1 |
| Total Packets Captured | 1,965 |
| Capture Date | 30 March 2026 |

# Protocol Background

## What is DNS?

DNS (Domain Name System) is like the internet's phone book. When you visit a website, your device first asks a DNS server to translate the domain name (e.g. spotify.com) into an IP address it can connect to. Monitoring DNS traffic is a core skill for SOC analysts because:

- It reveals what services a device is communicating with

- Malware often uses DNS to reach command-and-control (C2) servers

- Unusual DNS queries can be an early indicator of compromise

## What is HTTP?

HTTP (Hypertext Transfer Protocol) is the protocol used to transfer data between a client and a web server. Analysing HTTP traffic allows an analyst to:

- See what files or resources are being requested

- Identify what servers a device is communicating with

- Detect unencrypted data transfers that could expose sensitive information

# How the Capture Was Done

1. Opened Wireshark on Windows

2. Selected the active WiFi interface

3. Started a live capture

4. Applied display filters: dns, http, tcp.stream eq 17

5. Observed and analysed the traffic between the laptop and external servers

# Device Identification

By analysing source and destination IP addresses:

- 172.20.10.1 — The router/gateway. Answers all DNS queries, acting as the DNS middleman.

- 172.20.10.5 — My laptop. Sends all DNS queries and initiates HTTP connections.

- 34.104.35.123 — External Google CDN server (edgedl.me.gvt1.com)

DNS Traffic Flow:

Laptop (172.20.10.5)

↓  "What is the IP for android.clients.google.com?"

Router (172.20.10.1)

↓  forwards query to upstream DNS

↑  returns the answer

Laptop connects to the resolved IP

# Section 1 — DNS Traffic Analysis

## Screenshot 1: DNS Filter — Full Packet List

The dns filter was applied to isolate all DNS traffic. The capture shows my laptop (172.20.10.5) querying the router (172.20.10.1) for multiple domains across different services.

## Domains Identified

| Domain | Service | Query Type |
| --- | --- | --- |
| metrics.icloud.com | Apple iCloud | A |
| android.clients.google.com | Google Android Services | A, HTTPS |
| safebrowsing.googleapis.com | Google Safe Browsing | HTTPS, A |
| settings-win.data.microsoft.com | Microsoft Windows Settings | A |
| login.microsoftonline.com | Microsoft Login/Auth | A |
| fd.api.iris.microsoft.com | Microsoft Iris API | A |

## Screenshot 2: DNS Packet with Full Frame Breakdown

This screenshot shows a selected DNS packet (Frame 788) with full breakdown visible, showing the protocol stack: eth:ethertype:ip:udp:dns

# Section 2 — HTTP Traffic Analysis

## Screenshot 3: HTTP Filter — Packet List

The http filter was applied to isolate HTTP traffic. The main activity visible is my laptop (172.20.10.5) communicating with 34.104.35.123 (Google Edge Cache — edgedl.me.gvt1.com).

Key observations:

- Packet 119 — HTTP GET request sent from laptop to server

- Packet 126 — TCP Previous Segment Not Captured (packet loss)

- Packets 129, 132, 133 — TCP Out-Of-Order (network congestion)

- Packets 136–158 — HTTP Continuation (large file transferred in chunks)

## Screenshot 4: Ethernet & IP Layer Expanded

- Source MAC: Intel_d8:83:9e (my laptop's network card)

- Destination MAC: ee:2c:73:c0:a4:64 (router)

- Source IP: 172.20.10.5 → Destination IP: 34.104.35.123

- TTL: 128 (typical Windows default)

## Screenshot 5: TCP Layer Expanded

- Source Port: 52825 → Destination Port: 80 (HTTP)

- Flags: PSH, ACK

- TCP Segment Length: 311 bytes

- Conversation: Complete, WITH_DATA

## Screenshot 6: HTTP Layer Expanded

The HTTP layer reveals the full GET request:

GET /edgedl/diffgen-puffin/...

User-Agent: Microsoft BITS/7.8

Host: edgedl.me.gvt1.com

Connection: Keep-Alive

Microsoft BITS is a Windows component that silently downloads updates in the background — confirming the laptop was downloading a Windows update via Google's CDN without user interaction.

## Screenshot 7: TCP Stream — Full HTTP Conversation

Follow TCP Stream on Stream 17 shows the full conversation:

- White text = request sent from laptop (GET request)

- Blue text = response from server

Server Response:

HTTP/1.1 200 OK

server: Google-Edge-Cache

content-length: 57220

content-type: application/octet-stream

last-modified: Mon, 16 Mar 2026

The garbled text below is raw binary file content — the actual Windows update file displayed as ASCII.

## Screenshot 8: Mixed Protocol Overview

Unfiltered view showing all active protocols during capture:

- DNS — Domain resolution

- TCP — Connection management

- TLSv1.2 — Encrypted HTTPS traffic

- QUIC — Google's modern transport protocol

Notable packets:

- Packet 779 — TCP RST/ACK (connection forcefully reset — red)

- Packet 780 — TCP Retransmission (lost packet resent)

- QUIC to 142.250.178.14 — Google services

# Key Findings

- Background traffic is constant — Device communicated with Apple, Google, and Microsoft without any apps actively open.

- Microsoft BITS used Google's CDN — Windows update downloaded through Google Edge Cache servers via unencrypted HTTP (port 80).

- TCP anomalies detected — Out-of-order packets and retransmissions indicate minor network congestion.

- QUIC protocol observed — Modern Google services bypass traditional TCP+TLS, making deep inspection harder.

- Unencrypted HTTP transfer — The update download used plain HTTP — flagworthy in a security audit.

# SOC Analyst Perspective

| Observation | Analyst Note |
| --- | --- |
| DNS to Google/Apple/Microsoft | Normal background activity — benign |
| HTTP download via BITS | Verify file hash — BITS can be abused by malware (LOLBin technique) |
| TCP Out-Of-Order/Retransmission | Minor congestion — not a threat here |
| QUIC traffic to Google | Normal for Chrome/Android — encrypted, harder to inspect |
| RST packet (Packet 779) | Forceful connection termination — worth investigating in real SOC |

# Skills Demonstrated

- Live network traffic capture with Wireshark

- Display filter usage (dns, http, tcp.stream eq 17)

- Protocol layer analysis (Ethernet, IP, TCP, HTTP, DNS)

- TCP Stream following and HTTP request/response analysis

- Device identification from IP and MAC addresses

- Security-relevant interpretation of network behaviour

OLAMIDE  |  Cybersecurity Student  |  TryHackMe SOC Level 1 Path